OWR-108 4G outdoor router

(V1.04)

EX-LINK OWR-108 Cellular outdoor router is a wireless communication product developed based on 4G LTE/5G network requirements. Supports 4G LTE global frequency band. Supports 4G LTE CAT 6, CAT 12 Internet access. The maximum downlink bandwidth of 4G LTE is 300Mbps~600Mbps, and the maximum uplink bandwidth is 50Mbps~150Mbps. It supports 2 Gigabit Ethernet ports. The equipment is designed to meet waterproof and dustproof requirements. Protection class IP65. providing stable and high-speed wireless broadband access for home and business users. Optional support for WIFI 2.4GHz/5GHz dual-band function to meet the access and sharing needs of more than 60 users' WIFI hotspots. Supports GPS positioning function, which is convenient for users to monitor operation and maintenance.

TYPICAL APPLICATION

Kiosk

Industrial  Automation

Transportation

ATM

Video

Surveillance

Broadband

HOR204

Outdoor router

GPS/BD

Wi-Fi Client

3G/4G/5G

LAN

Management

Platform

MaxComm Co.,Ltd  www.4gltecpe.com email:sales@maxcomm.com.tw

eWebsite：www.ex-link.cn ,     TEL: +86-755-86111909,       FAX: +86-755-86111900 ,    E_mail: sales@ex-link-cn

Product features

◆ Wireless Mobile Broadband 3G / 4G Connection. Supports 3G/4G global frequency bands.

◆ Supports 4G LTE CAT 6, CAT 12 high speed internet access.

◆ System crashes and automatically recovers. The system automatically maintains the data link and is permanently online.

◆ Supports Multiple VPN tunnels for data encryption.

Designed for industrial usage

◆ Power Input Range. POE DC +12V to +36 VDC

◆ Industrial design for harsh environment.

◆ ABS fireproof plastic material shell.

Easy to use and easy maintenance

◆ User friendly web interface for user interaction.

◆ Supports Central Management Platform.

◆ Supports  local web UI and remote FOTA update firmware.

Operating system

◆ Built-in OpenWRT 18.06 operating system. Supports user secondary application development.

SPECIFICATIONS

| Cellular feature – 4G LTE CAT 6，CAT 12 Version |  |
| --- | --- |
| 3G/4G data connectivity | It provides data connectivity on LTE-FDD, LTE-TDD, DC-HSDPA, HSPA+, HSDPA, HSUPA and WCDMA networks. |
| 3G/4G Frequency bands | Version E. LTE-FDD: B1/B3/B5/B7/B8/B20/B28/B32(Optional) LTE-TDD: B38/B40/B41/B42(Optional) /B43(Optional) WCDMA: B1/B3/B5/B8 Verison A. LTE-FDD: B2/B4/B5/B7/B12/B13/B14B25/B26/B29/B30/B66/B71 LTE-TDD: B41/B48 Version LA. LTE-FDD: B2/B4/B5/B7/B8/B28/B66 LTE-TDD: B42(Optional) /B43(Optional) WCDMA: B2/B4/B5/B8 Version GL. LTE-FDD: B1/B2/B3/B4/B5/B7/B8/B12/B13/B14/B17/B18/B19/B20/B25/B26/B28/B29/B30/B32/B66/B71 LTE-TDD: B34/B38/B39/B40/B41/B42/B43/B46 (LAA)/B48 (CBRS) WCDMA: B1/B2/B3/B4/B5/B6/B8/B19 Remarks. More frequency bands or requirementes please contact with us. |
| 3G/4G Data rates | 4G LTE--CAT 6 LTE-FDD Max 300M(DL) , Max 50Mbps (UL) LTE-TDD Max 226M (DL) , Max 28Mbps (UL). 4G LTE-CAT 12 LTE-FDD. Max 600Mbps (DL)，Max 150Mbps (UL) . LTE-TDD. Max 430Mbps (DL)，Max 90Mbps (UL). HSPA+. DL 42Mbps, UL 11.2Mbps. WCDMA. DL 384Kbps, UL 384Kbps |
| 3G/4G Transmitting Power | ◆ Class 3 (23 dBm±2 dB) for LTE-TDD bands ◆ Class 3 (23 dBm±2 dB) for LTE-FDD bands ◆ Class 3 (24 dBm+ 1/-3 dB) for WCDMA bands |
| 3G/4G LTE Receiving Sensitivity | WCDMA B1 , -106.7 dBm WCDMA B2 , -104.7 dBm WCDMA B3 , -103.7 dBm WCDMA B4 , -106.7 dBm WCDMA B5 , -104.7 dBm WCDMA B6 , -106.7 dBm WCDMA B8 , -103.7 dBm WCDMA B19 , -106.7 dBm LTE-FDD B1(10MHz) , -96.3 dBm LTE-FDD B2(10MHz) , -94.3 dBm LTE-FDD B3(10MHz) , -93.3 dBm LTE-FDD B4(10MHz) , -96.3 dBm LTE-FDD B5(10MHz) , -94.3 dBm LTE-FDD B7(10MHz) , -94.3 dBm LTE-FDD B8(10MHz) , -93.3 dBm LTE-FDD B12(10MHz) , -93.3 dBm LTE-FDD B13(10MHz) , -93.3 dBm LTE-FDD B14(10MHz) , -93.3 dBm LTE-FDD B17(10MHz) , -93.3 dBm LTE-FDD B18(10MHz) , -96.3 dBm LTE-FDD B19(10MHz) , -96.3 dBm LTE-FDD B20(10MHz) , -96.3 dBm LTE-FDD B25(10MHz) , -92.8 dBm LTE-FDD B26(10MHz) , -93.8 dBm LTE-FDD B28(10MHz) , -94.8 dBm LTE-FDD B30(10MHz) , -95.3 dBm LTE-FDD B32(10MHz) , -96.3 dBm LTE-FDD B66(10MHz) , -95.8 dBm LTE-FDD B71(10MHz) , -94.3 dBm LTE-TDD B38(10MHz) , -96.3 dBm LTE-TDD B39(10MHz) , -96.3 dBm LTE-TDD B40(10MHz) , -96.3 dBm LTE-TDD B41(10MHz) , -94.3 dBm LTE-TDD B42(10MHz) , -95 dBm LTE-TDD B43(10MHz) , -95 dBm LTE-TDD B46(20MHz) , -88.5 dBm LTE-TDD B48(10MHz) , -95 dBm |
| LTE features | CAT 6 version. ◆ Supports 3GPP Rel-12 CAT 6 FDD and TDD standard. ◆ Supports uplink QPSK and 16 QAM modulation. ◆ Supports downlink QPSK , 16 QAM and 64 QAM. ◆ Supports 1.4MHz to 40MHz (2 CA) RF bandwidth. CAT 12 version. ◆ Supports 3GPPRel-12 CAT 12 FDD and TDD standard. ◆ Supports uplink QPSK ,16 QAM and 64 QAM modulation. ◆ Supports downlink QPSK , 16 QAM ,64 QAM and 256 QAM. ◆ Supports 1.4MHz to 60MHz (3 CA) RF bandwidth. |
| UMTS features | ◆ Supports 3GPP Rel-9 DC-HSDPA,DC-HSUPA,HSPA+,HSDPA,HSUPA and WACDMA ◆ Supports QPSK ,16 QAM and 64 QAM modulation. |
| Antennas | ◆ 2*2 MIMO 4G internal antennas or 4*4 MIMO 4G internal antenna, with 50 Ω impedance. ◆ 1* GPS antenna.(Optional). |
| GNSS/GPS (Optional) |  |
| GNSS features | ◆ Gen9HT of Qualcomm ◆ Protocol: NMEA 0183, Data Update Rate: 1Hz. ◆ GPS,GLONASS,BeiDou(COMPASS)/Galileo. ◆ GPS/Galileo/QZSS.1575.42 ±1.023 (L1) MHz ◆ Galileo.1575.42 ±2.046 (E1) MHz ◆ QZSS. 1575.42 (L1) MHz ◆ GLONASS. 1597.5–1605.8 MHz ◆ BeiDou. 1561.098 ±2.046MHz |
| SIM card feature |  |
| SIM card | Supports 1* SIM slots, 1.8V / 3V . Or 1* eSIM card. ( Optional ) |
| Hardware feature |  |
| CPU | MTK7621A, MIPS1004KC, 880Mbps,Dual-core. |
| 4G Modem | Qualcomm SDX12 |
| MEMORY | FLASH 16MByte, DDR3 64MByte |
| Hardware interface | 2* LAN gigabit Ethernet ports. 1*USB2.0 port. |
| Watchdog | Built-in watchdog feature. |
| Key button | Reset |
| Protection level | Ethernet port , Contact electric shock, +/-4KV, air discharge: +/-8KV. |
| LED status indicator | PWR ,SYS, Net, WAN, LAN,WLAN |
| Standard Power | POE power supply input. DC 24V/1A |
| Power Supply | POE power supply input. DC +12/1.5A~36V/0.5A |
| Peak current | Max current. 2.5A |
| Working current | Max 360 mA,2.15W @24 V |
| Power Consumption | Idle. 40mA, 0.96W @24 V Data link. Max 180 mA, 2.15W @24 V Peak. Max 235mA , 3.2W @24V |
| Temperature | Operating Temperature -20~+60ºC, Storage Temperature -30~+70ºC |
| Environment humidty | 5%~95% ,no condensation. |
| Ingress Protection | IP65 |
| Housing | ABS fireproof plastic material, white shell. |
| Dimensions | 250mm * 145mm * 67.5mm |
| Installations | Wall or pole mounted. |
| Weight | 484g |
| Wi-Fi （Optional） |  |
| WLAN | ◆ IEEE 802.11b/g/n/ac. ◆ Supports 20MHz,40MHz,80MHz in 5GHz band, and 20MHz,40MHz bandwidth in 2.4GHz band. ◆ Max bandwidth 300Mbps in 2.4GHz band. Max bandwidth 867Mbps in 5GHz band. |
| Wireless Mode | Access Point (AP), Client |
| Wireless Speed | 300Mbps @2.4GHz, 867Mbps @ 5GHz. |
| Wireless Security | Supports WPA, WPA2,WPAI, WEP, TKIP encryption. |
| Frequency Bands | 2.4 GHz/5GHz |
| WIFI transmit power | 2.4GHz Tx power. TX CCK, 11Mbps @ -20dBm HT40,MCS 8 @ -20dBm HT40,MCS 15 @ -17dBm 5GHz Tx power. 6Mbps OFDM @ -19.5dBm 54Mbps OFDM @ -18dBm HT20,MCS 0 @ -18.5dBm HT20,MCS 7 @ -17dBm HT40,MCS 0 @ -18.5dBm HT40,MCS 7 @ -17dBm VHT80,MCS 0 @ -18.5dBm VHT80,MCS 9 @ -16dBm |
| WIFI Rx Sensitivity | 2.4GHz Rx sensitivity. 1Mbps CCK @ -98dBm 2Mbps CCK @ -94dBm 5.5Mbps CCK @ -92dBm 11Mbps CCK @ -89dBm 5GHz Rx sensitivity. 6Mbps OFDM @ -93.5dBm 9Mbps OFDM @ -91.5dBm 12Mbps OFDM @ -91dBm 18Mbps OFDM @ -88.5dBm 24Mbps OFDM @ -84.5dBm 36Mbps OFDM @ -82dBm 48Mbps OFDM @ -77dBm 54Mbps OFDM @ -76dBm |
| Antennas | 2*2 MIMO internal antennas, with 50 Ω impedance. |
| WIFI hotspot sharing | Supports more than 60 users to share WIFI access to the Internet. |
| Software feature |  |
| Parameter settings | Supports automatic detection of MNC and MCC parameters of global operators. Built-in global operator APN, user name, password and other network parameters. At the same time, manual setting of network parameters is supported. |
| Dial method | After the device is powered on, the system automatically dials up to connect to the network. |
| Protocol | Supports PPTP,L2TP,IPSEC VPN,TCP,UDP,DHCP,HTTP,DDNS,TR-069,HTTPS,SSH,SNMP etc protocol. |
| Routing | Supports static routing, multiple routing tables. |
| Bridge | Supports 4G bridge mode feature. |
| Multiple APN | Supports multiple APN access network. |
| System Assurance | Supports system automatic detection mechanism, automatic recovery of system abnormality or crash. |
| Data Link Assurance | Built-in data link maintenance and self-recovery mechanism. |
| Firewall | Support flexible access control of TCP, UDP, ICMP packets. Supports port Mapping, NAT etc feature. |
| DDNS | Supported some service providers, others can be configured manually. |
| Firmware update | Supports local WebUI and remote OTA update firmware. |
| VLAN | Supports VLAN feature. |
| Application development | Supports secondary development of application functions based on our device motherboard software. |
| VPN |  |
| VPN Feature | Supports OpenVPN, IPSEC VPN,PPTP,L2TP etc VPN feature. |
| MONITORING & MANAGEMENT |  |
| Web GUI | HTTP, Firmware Upgrade |
| Command Line Interface | SSHv2 ,telnet |
| Management | Remote Management |

Dimensions.